9.  Playing out to tape in off-line

## PLAYING OUT TO VHS

Put a tape in the VHS machine.  Load up your Project and open your Program.

Watch the loudest part of the piece ensuring the audio levels on the VHS player do not peak higher that about 4 (in the red).

Place the play line at the beginning of the Program.

Press Record on the VHS machine, wait about 5seconds and then simply Play the Program.

### THE PATCH PANEL

In the main digitising suits it is necessary to tell the computer where you are digitising from / playing out to.

You do this with the Patch Panel, which works in the same way as those in the Transfer Suite.

To Digitise:

You have to patch up the panel correctly to be able to digitise, so make sure:

- One cable goes From the tape deck you are using To Media 100 in.  If it is Beta or DVC-Pro you are digitising from ensure the cable goes from Clean.

- The second goes From the tape deck you are using To Monitor.  If it is Beta or DVC-Pro you are digitising from ensure the cable goes from BITC.

- If you are using DVC-Pro or BETA a third cable must be connected From the appropriate timecode To timecode to digitise.

- If you want to watch what you are editing on the monitor (i.e. if you wish to check a clip you have digitised) you must change the second cable to go From Media 100, To the monitor.

To Play Out:

You have to patch up the panel correctly to be able to play out, so make sure:

- One cable must go From Media 100 To the VHS player.

- The second cable goes From the VHS player To the monitor, so you can check you are playing out correctly.